Remarks on Final Draft ETSI ES 201 671 V2.1.1 (2001-07)

- §3.2 Abbreviations

- Add MO (Mobile Originated) and MT (Mobile Terminated)

- §8.1  Data transmission protocols

- “The protocol used by   … based on existing protocols like ROSE or FTP” makes one suspect other protocols may also be used.

- §8.2  Application for IRI

- “… if LI is being activated during an already established communication ….” 
This is a general topic, not specific for packet-oriented data services such as GPRS.  If a new LI measure immediately applies to already established communications (optional ) the IRI-BEGIN record for this communication is created at LI activation time.

- “If LI is being deactivated during an established communication, no IRI-END record will be transmitted.”  
What speaks against this IRI-END record ? It can be made optional, but should not be forbidden.

- §11.1  Security properties

- definition of  “availability”

- Availability is not really a security aspect, but rather an aspect of “Performance and Quality”. This is already reflected in the fact that availability is not mentioned again in §11.2.

- Paragraph is to long. Not everything belongs to “availability”

- Proposal for structure : 
“A secure communication channel has the following properties : 
Confidentiality, Integrity, Authentication  (, Availability). 
Confidentiality means …
Confidentiality can be an issue ….
Integrity means …. 
Integrity can be an issue …
Authentication means ….
Authentication can be an issue ….   In the process of LI it is very important ….
(Availability means …. 
Availability can be an issue ….)
Irregularities …”

- §A.3.3   Selection of parameters for IRI records

- “Three types of origins for the specification of HI2 …”
has 4 numbered paragraphs below. The third must not have an own number.

- §A.4  HI3: interface port for Content of Communication

- First three lines are copies from §9. Duplicated information is bad for document maintenance.

- Since “mono transmission” is no longer mentioned, best remove the term “stereo transmission” also.

- §A.4.3  Control information for CS-CC

- First paragraph beneath table A.4.1, description of parameter 5 : 
“… in case of separate transmission of each communication direction … “ can be removed since “mono transmission” is no longer mentioned.