3GPP LI 	Tdoc S3LI01_74

Saarbruken August 21-23

| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 33.107 |  |  |  | CR | xxx |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 5.00 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  |  |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  | X |
| Title:  |  | Source of PDP context initiation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Motorola |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | Security |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | Aug 13, 2001 |  |  |  |  |  |
| Category:  |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | The “Start of interception of active PDP context ” and “PDP context activation ” reports, provide no indication regarding the source of the PDP context initiation, i.e. whether the MS has initiated the PDP session it or whether it was done by the network. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | The PDU Notification Request as defined in 3GPP TS 23.060 is being added to this specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | Missing capability and compatibility with 3GPP TS 23.060. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 7.3, 7.3.2, 7.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://www.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2000-09 contains the specifications resulting from the September 2000 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

## 7.3	Provision of Intercept Related Information

Intercept Related Information (Events) are necessary at the Mobile Station Attach, Mobile Station Detach, PDU Notification Request, PDP Context Activation, Start of intercept with PDP context active, PDP Context Deactivation, RA update, and SMS events.

### 7.3.2	Structure of the events

There are seven different events in which the information is sent to the DF2 if this is required. Details are described in the following section. The events for interception are configurable (if they are sent to DF2) in the 3G GSN and can be suppressed in the DF2.

The following events are applicable to 3G SGSN:

-	Mobile Station Attach;

- Mobile Station Detach;

- PDU Notification Request

-	PDP context activation;

-	Start of intercept with PDP context active;

-	PDP context deactivation;

-	RA update;

-	SMS.

## 7.4	Packet Data related events

### 7.4.1	Mobile Station Attach

For attach an attach-event is generated. When an attach activation is generated from the mobile to servicing 3G G SN this event is generated. These fields will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Location Information

Failed attach reason

IAs (if applicable)

### 7.4.2	Mobile Station Detach

For detach a detach-event is generated, this is for the common (end) detach. These fields will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Location Information

IAs (if applicable)

### 7.4.3	PDU Notification Request

For PDU Notification request a  PDU Notification -event is generated. The following fields will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

PDP address of observed party

Event Type

Event Time

Event Date

Access Point Name

PDP Type

Network Element Identifier

Location Information

IAs (if applicable)

### 7.4.4	Packet Data PDP context activation

For PDP context activation a PDP context activation-event is generated. When a PDP context activation is generated from the mobile to 3G GSN this event is generated. These fields will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

PDP address of observed party

Event Type

Event Time

Event Date

Correlation number

Access Point Name

PDP Type

Network Element Identifier

Location Information

Failed context activation reason

IAs (if applicable)